IP FABRICS: COLLAPSED CORE

Anurag Shekhar

TOPOLOGY

RFC:

https://datatracker.ietf.org/doc/html/rfc8365

# [Chapter 1] — COLLAPSED CORE CAMPUS FABRIC

This is one of the models offered by HPE Juniper for campus use. As the name suggest collapsed core means, we have one layer that works both as a core and aggregation/distribution. This model is quite simple and have only two layers collapsed core and access. All the L3 is in the collapsed core switch while the access switches are pure L2. Also note that to achieve load balancing, redundancy and smooth movement of users across the location in the campus we will use ESI-LAG, each access switches are multihomed to all the spine switches.

The main work will only be done on the two spine switches. Please check the above diagram to get parameters for this design. Let us start the build.

SPINE 1 CONFIG:

---------------------------------------------------------------------------------------------------------------------

EBGP UNDERLAY : THE PRIMARY PURPOSE IS TO MAKE THE LO0 OF BOTH THE SPINES REACHABLE : PING SHOULD BE SUCCESSFUL

[edit]

interfaces {

These are the L3 interfaces that is connecting the two spines. We want to connect them to avoid split brain behavior

ge-0/0/0 {

unit 0 {

family inet {

address 16.0.0.1/30;

}

}

}

ge-0/0/1 {

unit 0 {

family inet {

address 17.0.0.1/30;

}

}

}

Best practice to keep the lo0 and Router id same

lo0 {

unit 0 {

family inet {

address 1.1.1.1/32;

}

}

}

}

routing-options {

router-id 1.1.1.1;

autonomous-system 2000;

}

[edit]

Note that EBGP import everything but do not export anything by default. We only want to advertise/export lo0 in underlay for security reasons and that is why we must set export policy

protocols {

bgp {

group EBGP_UNDERLAY {

type external;

peer-as 6000;

local-as 5000;

export ADV-LO0;

neighbor 16.0.0.2 {

local-address 16.0.0.1;

}

neighbor 17.0.0.2 {

local-address 17.0.0.1;

}

}

}

Export policy using route filter to only advertise/export lo0 addresses

}

policy-options {

policy-statement ADV-LO0 {

term 1 {

from {

protocol direct;

route-filter 1.1.1.1/32 exact;

}

then accept;

}

}

}

---------------------------------------------------------------------------------------------------------------------

VALIDATION

jcluser@spine1> show bgp summary

Warning: License key missing; requires 'bgp' license

Threading mode: BGP I/O

Default eBGP mode: advertise - accept, receive - accept

Groups: 1 Peers: 2 Down peers: 0

Table          Tot Paths  Act Paths Suppressed    History Damp State    Pending

inet.0

2          1          0          0          0          0

Peer                     AS      InPkt     OutPkt    OutQ   Flaps Last Up/Dwn State|#Active/Received/Accepted/Damped...

16.0.0.2               6000         49         47       0       0       20:46 Establ

inet.0: 1/1/1/0

17.0.0.2               6000         49         47       0       0       20:42 Establ

inet.0: 0/1/1/0

As you can see above, both EBGP peering have been established.

jcluser@spine1> show route

inet.0: 6 destinations, 7 routes (6 active, 0 holddown, 0 hidden)

Limit/Threshold: 1048576/1048576 destinations

+ = Active Route, - = Last Active, * = Both

1.1.1.1/32         *[Direct/0] 00:31:41

>  via lo0.0

2.2.2.2/32         *[BGP/170] 00:11:08, localpref 100

AS path: 6000 I, validation-state: unverified

>  to 16.0.0.2 via ge-0/0/0.0

[BGP/170] 00:11:08, localpref 100

AS path: 6000 I, validation-state: unverified

>  to 17.0.0.2 via ge-0/0/1.0

16.0.0.0/30        *[Direct/0] 00:31:41

>  via ge-0/0/0.0

16.0.0.1/32        *[Local/0] 00:31:41

Local via ge-0/0/0.0

17.0.0.0/30        *[Direct/0] 00:31:41

>  via ge-0/0/1.0

17.0.0.1/32        *[Local/0] 00:31:41

Local via ge-0/0/1.0

mgmt_junos.inet.0: 3 destinations, 4 routes (3 active, 0 holddown, 0 hidden)

+ = Active Route, - = Last Active, * = Both

0.0.0.0/0          *[Static/5] 00:51:04

>  to 100.123.0.1 via fxp0.0

[Access-internal/12] 00:51:03, metric 0

>  to 100.123.0.1 via fxp0.0

100.123.0.0/16     *[Direct/0] 00:51:04

>  via fxp0.0

100.123.51.1/32    *[Local/0] 00:51:04

Local via fxp0.0

If you see above we have now routes to the loopbacks of both the spines. 1.1.1.1/32 is the local lo0, so it is direct and the spine 2 lo0 2.2.2.2/32 has two paths of same local preference of 100, this effectively load balance the traffic between the two links. Also note the 6000 I, it means you are going from one AS to another.

jcluser@spine1> show route advertising-protocol bgp 16.0.0.2

Warning: License key missing; requires 'bgp' license

inet.0: 6 destinations, 7 routes (6 active, 0 holddown, 0 hidden)

Limit/Threshold: 1048576/1048576 destinations

Prefix                  Nexthop              MED     Lclpref    AS path

* 1.1.1.1/32              Self

jcluser@spine1> show route advertising-protocol bgp 17.0.0.2

Warning: License key missing; requires 'bgp' license

inet.0: 6 destinations, 7 routes (6 active, 0 holddown, 0 hidden)

Limit/Threshold: 1048576/1048576 destinations

Prefix                  Nexthop              MED     Lclpref    AS path

* 1.1.1.1/32              Self

This command show that what your router is exporting to its neighbour, as expected it is exporting the lo0 of the spine 1

jcluser@spine1> show route receive-protocol bgp 16.0.0.2

Warning: License key missing; requires 'bgp' license

inet.0: 6 destinations, 7 routes (6 active, 0 holddown, 0 hidden)

Limit/Threshold: 1048576/1048576 destinations

Prefix                  Nexthop              MED     Lclpref    AS path

* 2.2.2.2/32              16.0.0.2                                6000 I

mgmt_junos.inet.0: 3 destinations, 4 routes (3 active, 0 holddown, 0 hidden)

jcluser@spine1> show route receive-protocol bgp 17.0.0.2

Warning: License key missing; requires 'bgp' license

inet.0: 6 destinations, 7 routes (6 active, 0 holddown, 0 hidden)

Limit/Threshold: 1048576/1048576 destinations

Prefix                  Nexthop              MED     Lclpref    AS path

2.2.2.2/32              17.0.0.2                                6000 I

mgmt_junos.inet.0: 3 destinations, 4 routes (3 active, 0 holddown, 0 hidden)

This command show that what your router is receiving from its neighbour, as expected it is receiving the lo0 of the spine 2

SPINE 2 CONFIG:

These are the L3 interfaces that is connecting the two spines. We want to connect them to avoid split brain behavior

[edit]

interfaces {

ge-0/0/0 {

unit 0 {

family inet {

address 16.0.0.2/30;

}

}

}

ge-0/0/1 {

unit 0 {

family inet {

address 17.0.0.2/30;

}

}

Best practice to keep the lo0 and Router id same

}

lo0 {

unit 0 {

family inet {

address 2.2.2.2/32;

}

}

}

}

routing-options {

router-id 2.2.2.2;

autonomous-system 2000;

}

protocols {

bgp {

group EBGP_UNDERLAY {

Note that EBGP import everything but do not export anything by default. We only want to advertise/export lo0 in underlay for security reasons and that is why we must set export policy

type external;

peer-as 5000;

local-as 6000;

export ADV-LO0;

neighbor 16.0.0.1 {

local-address 16.0.0.2;

}

neighbor 17.0.0.1 {

local-address 17.0.0.2;

}

}

}

}

Export policy using route filter to only advertise/export lo0 addresses

policy-options {

policy-statement ADV-LO0 {

term 1 {

from {

protocol direct;

route-filter 2.2.2.2/32 exact;

}

then accept;

}

}

}

---------------------------------------------------------------------------------------------------------------------

VALIDATION

---------------------------------------------------------------------------------------------------------------------

jcluser@spine2> show bgp summary

Warning: License key missing; requires 'bgp' license

Threading mode: BGP I/O

Default eBGP mode: advertise - accept, receive - accept

Groups: 1 Peers: 2 Down peers: 0

Table          Tot Paths  Act Paths Suppressed    History Damp State    Pending

inet.0

2          1          0          0          0          0

Peer                     AS      InPkt     OutPkt    OutQ   Flaps Last Up/Dwn State|#Active/Received/Accepted/Damped...

16.0.0.1               5000         77         76       0       0       33:16 Establ

inet.0: 1/1/1/0

17.0.0.1               5000         77         76       0       0       33:12 Establ

inet.0: 0/1/1/0

As you can see above, both EBGP peering have been established.

jcluser@spine2> show route

inet.0: 6 destinations, 7 routes (6 active, 0 holddown, 0 hidden)

Limit/Threshold: 1048576/1048576 destinations

+ = Active Route, - = Last Active, * = Both

1.1.1.1/32         *[BGP/170] 00:28:04, localpref 100

AS path: 5000 I, validation-state: unverified

>  to 16.0.0.1 via ge-0/0/0.0

[BGP/170] 00:28:04, localpref 100

AS path: 5000 I, validation-state: unverified

>  to 17.0.0.1 via ge-0/0/1.0

2.2.2.2/32         *[Direct/0] 00:44:17

>  via lo0.0

16.0.0.0/30        *[Direct/0] 00:44:17

>  via ge-0/0/0.0

16.0.0.2/32        *[Local/0] 00:44:17

Local via ge-0/0/0.0

17.0.0.0/30        *[Direct/0] 00:44:17

>  via ge-0/0/1.0

17.0.0.2/32        *[Local/0] 00:44:17

Local via ge-0/0/1.0

mgmt_junos.inet.0: 3 destinations, 4 routes (3 active, 0 holddown, 0 hidden)

+ = Active Route, - = Last Active, * = Both

0.0.0.0/0          *[Static/5] 01:03:28

>  to 100.123.0.1 via fxp0.0

[Access-internal/12] 01:03:28, metric 0

>  to 100.123.0.1 via fxp0.0

100.123.0.0/16     *[Direct/0] 01:03:28

>  via fxp0.0

100.123.51.2/32    *[Local/0] 01:03:28

Local via fxp0.0

If you see above we have now routes to the loopbacks of both the spines. 2.2.2.2/32 is the local lo0, so it is direct and the spine 1 lo0 1.1.1.1/32 has two paths of same local preference of 100, this effectively load balance the traffic between the two links. Also note the 5000 I, it means you are going from one AS to another.

jcluser@spine2> show route advertising-protocol bgp 16.0.0.1

Warning: License key missing; requires 'bgp' license

inet.0: 6 destinations, 7 routes (6 active, 0 holddown, 0 hidden)

Limit/Threshold: 1048576/1048576 destinations

Prefix                  Nexthop              MED     Lclpref    AS path

* 2.2.2.2/32              Self                                    I

jcluser@spine2> show route advertising-protocol bgp 17.0.0.1

Warning: License key missing; requires 'bgp' license

inet.0: 6 destinations, 7 routes (6 active, 0 holddown, 0 hidden)

Limit/Threshold: 1048576/1048576 destinations

Prefix                  Nexthop              MED     Lclpref    AS path

* 2.2.2.2/32              Self                                    I

This command show that what your router is exporting to its neighbour, as expected it is exporting the lo0 of the spine 2

jcluser@spine2> show route receive-protocol bgp 16.0.0.1

Warning: License key missing; requires 'bgp' license

inet.0: 6 destinations, 7 routes (6 active, 0 holddown, 0 hidden)

Limit/Threshold: 1048576/1048576 destinations

Prefix                  Nexthop              MED     Lclpref    AS path

* 1.1.1.1/32              16.0.0.1                                5000 I

mgmt_junos.inet.0: 3 destinations, 4 routes (3 active, 0 holddown, 0 hidden)

jcluser@spine2> show route receive-protocol bgp 17.0.0.1

Warning: License key missing; requires 'bgp' license

inet.0: 6 destinations, 7 routes (6 active, 0 holddown, 0 hidden)

Limit/Threshold: 1048576/1048576 destinations

Prefix                  Nexthop              MED     Lclpref    AS path

1.1.1.1/32              17.0.0.1                                5000 I

mgmt_junos.inet.0: 3 destinations, 4 routes (3 active, 0 holddown, 0 hidden)

This command show that what your router is receiving from its neighbour, as expected it is receiving the lo0 of the spine 1

---------------------------------------------------------------------------------------------------------------------

IBGP OVERLAY : IT WILL ONLY BE SET BETWEEN THE LO0 OF BOTH THE SPINES

SPINE 1

WE ARE USING MP BGP WITH EVPN SIGNALING: THIS TYPE OF BGP WILL BE ABLE TO CARRY EVPN ATTRIBUTES

[edit protocols]

bgp {

group IBGP_OVERLAY {

type internal;

local-address 1.1.1.1;

family evpn {

signaling;

}

cluster 1.1.1.1;

neighbor 2.2.2.2;

}

}

jcluser@spine1> show bgp summary

Warning: License key missing; requires 'bgp' license

Threading mode: BGP I/O

Default eBGP mode: advertise - accept, receive - accept

Groups: 2 Peers: 3 Down peers: 0

Table          Tot Paths  Act Paths Suppressed    History Damp State    Pending

inet.0

2          1          0          0          0          0

bgp.evpn.0

24         24          0          0          0          0

Peer                     AS      InPkt     OutPkt    OutQ   Flaps Last Up/Dwn State|#Active/Received/Accepted/Damped...

2.2.2.2                2000       1264       1281       0       1     9:17:45 Establ

bgp.evpn.0: 24/24/24/0

default-switch.evpn.0: 22/22/22/0

__default_evpn__.evpn.0: 2/2/2/0

16.0.0.2               6000       1352       1348       0       0    10:09:19 Establ

inet.0: 1/1/1/0

17.0.0.2               6000       1351       1347       0       0    10:09:15 Establ

inet.0: 0/1/1/0

WE ARE USING MP BGP WITH EVPN SIGNALING: THIS TYPE OF BGP WILL BE ABLE TO CARRY EVPN ATTRIBUTES

SPINE 2

[edit protocols]

bgp {

group IBGP_OVERLAY {

type internal;

local-address 2.2.2.2;

family evpn {

signaling;

}

cluster 1.1.1.1;

neighbor 1.1.1.1;

}

}

jcluser@spine2> show bgp summary

Warning: License key missing; requires 'bgp' license

Threading mode: BGP I/O

Default eBGP mode: advertise - accept, receive - accept

Groups: 2 Peers: 3 Down peers: 0

Table          Tot Paths  Act Paths Suppressed    History Damp State    Pending

inet.0

2          1          0          0          0          0

bgp.evpn.0

24         24          0          0          0          0

Peer                     AS      InPkt     OutPkt    OutQ   Flaps Last Up/Dwn State|#Active/Received/Accepted/Damped...

1.1.1.1                2000       1286       1265       0       0     9:19:12 Establ

bgp.evpn.0: 24/24/24/0

default-switch.evpn.0: 22/22/22/0

__default_evpn__.evpn.0: 2/2/2/0

16.0.0.1               5000       1352       1353       0       0    10:10:45 Establ

inet.0: 1/1/1/0

17.0.0.1               5000       1352       1352       0       0    10:10:41 Establ

inet.0: 0/1/1/0

---------------------------------------------------------------------------------------------------------------------

VLAN VXLAN EVPN

There are two vlans named on my two-name Anurag and Shekhar. I have mapped them to their respective vnis

SPINE 1

vlans {

anurag {

vlan-id 10;

l3-interface irb.10;

vxlan {

vni 5010;

There is a lot about ingress node replication: Check it in official Juniper documentation. Related to flooding optimization and recommendation is not to use this keyword. If this keyword is added/removed from active network, outage will happen and the network will reconverge

ingress-node-replication;

}

}

shekhar {

vlan-id 20;

l3-interface irb.20;

vxlan {

vni 5020;

ingress-node-replication;

}

}

}

Route distinguisher tells that the routes advertised in from the switch with lo0 1.1.1.1.

Also vxlan encapsulation happens at the lo0 which is the vtep. This is the start of the vxlan tunnel

switch-options {

vtep-source-interface lo0.0;

route-distinguisher 1.1.1.1:2000;

vrf-target target:2000:100;

}

evpn {

encapsulation vxlan;

multicast-mode ingress-replication;

vni-options {

EVPN has many algorithms for encapsulation, we are using here is VXLAN.

vni 5010 {

vrf-target target:2000:5010;

}

vni 5020 {

vrf-target target:2000:5020;

}

}

extended-vni-list [ 5010 5020 ];

SPINE 2

vlans {

anurag {

vlan-id 10;

l3-interface irb.10;

vxlan {

vni 5010;

ingress-node-replication;

}

}

shekhar {

vlan-id 20;

l3-interface irb.20;

vxlan {

vni 5020;

ingress-node-replication;

}

}

}

switch-options {

vtep-source-interface lo0.0;

route-distinguisher 2.2.2.2:2000;

vrf-target target:2000:100;

}

evpn {

encapsulation vxlan;

multicast-mode ingress-replication;

vni-options {

vni 5010 {

vrf-target target:2000:5010;

}

vni 5020 {

vrf-target target:2000:5020;

}

}

extended-vni-list [ 5010 5020 ];

---------------------------------------------------------------------------------------------------------------------

SETTING UP ESI LAG

SPINE 1

jcluser@spine1> show configuration interfaces ge-0/0/2

gigether-options {

802.3ad ae0;

}

jcluser@spine1> show configuration interfaces ge-0/0/3

gigether-options {

802.3ad ae1;

}

Separate esi and lacp system id for each ae but it must match for same ae in different spine. These are in hex

jcluser@spine1> show configuration interfaces ae0

esi {

00:11:22:33:44:55:66:77:88:99;

all-active;

}

aggregated-ether-options {

lacp {

active;

system-id 00:00:00:11:11:11;

}

}

unit 0 {

family ethernet-switching {

interface-mode trunk;

vlan {

members [ anurag shekhar ];

}

}

}

jcluser@spine1> show configuration interfaces ae1

esi {

00:11:22:33:44:55:66:77:88:aa;

all-active;

}

aggregated-ether-options {

lacp {

active;

system-id 00:00:11:11:11:11;

}

}

unit 0 {

family ethernet-switching {

interface-mode trunk;

vlan {

members [ anurag shekhar ];

}

}

}

SPINE 2

jcluser@spine2> show configuration interfaces ge-0/0/2

gigether-options {

802.3ad ae0;

}

jcluser@spine2> show configuration interfaces ge-0/0/3

gigether-options {

802.3ad ae1;

}

jcluser@spine2> show configuration interfaces ae0

esi {

00:11:22:33:44:55:66:77:88:99;

all-active;

}

aggregated-ether-options {

lacp {

active;

system-id 00:00:00:11:11:11;

}

}

unit 0 {

family ethernet-switching {

interface-mode trunk;

vlan {

members [ anurag shekhar ];

}

}

}

jcluser@spine2> show configuration interfaces ae1

esi {

00:11:22:33:44:55:66:77:88:aa;

all-active;

}

aggregated-ether-options {

lacp {

active;

system-id 00:00:11:11:11:11;

}

}

unit 0 {

family ethernet-switching {

interface-mode trunk;

vlan {

members [ anurag shekhar ];

}

}

}

ACCESS 1

jcluser@vEX_NV> show configuration interfaces ae0

aggregated-ether-options {

lacp {

active;

}

}

unit 0 {

family ethernet-switching {

interface-mode trunk;

vlan {

members [ anurag shekhar ];

}

}

}

jcluser@vEX_NV> show configuration interfaces ge-0/0/1

gigether-options {

802.3ad ae0;

}

jcluser@vEX_NV> show configuration interfaces ge-0/0/2

gigether-options {

802.3ad ae0;

}

jcluser@vEX_NV> show configuration vlans

anurag {

vlan-id 10;

}

shekhar {

vlan-id 20;

}

ACCESS 2

jcluser@vEX_NV> show configuration interfaces ae1

aggregated-ether-options {

lacp {

active;

}

}

unit 0 {

family ethernet-switching {

interface-mode trunk;

vlan {

members [ anurag shekhar ];

}

}

}

jcluser@vEX_NV> show configuration interfaces ge-0/0/1

gigether-options {

802.3ad ae1;

}

jcluser@vEX_NV> show configuration interfaces ge-0/0/2

gigether-options {

802.3ad ae1;

}

jcluser@vEX_NV> show configuration vlans

anurag {

vlan-id 10;

}

shekhar {

vlan-id 20;

}

---------------------------------------------------------------------------------------------------------------------

ESI VALIDATION

SPINE 1

jcluser@spine1> show lacp interfaces

Aggregated interface: ae0

LACP state:           Role   Exp   Def  Dist  Col  Syn  Aggr  Timeout  Activity

ge-0/0/2           Actor    No    No   Yes  Yes  Yes   Yes     Fast    Active

ge-0/0/2         Partner    No    No   Yes  Yes  Yes   Yes     Fast    Active

LACP protocol:        Receive State  Transmit State          Mux State

ge-0/0/2                  Current   Fast periodic Collecting distributing

Aggregated interface: ae1

LACP state:           Role   Exp   Def  Dist  Col  Syn  Aggr  Timeout  Activity

ge-0/0/3           Actor    No    No   Yes  Yes  Yes   Yes     Fast    Active

ge-0/0/3         Partner    No    No   Yes  Yes  Yes   Yes     Fast    Active

LACP protocol:        Receive State  Transmit State          Mux State

ge-0/0/3                  Current   Fast periodic Collecting distributing

jcluser@spine1> show interfaces esi terse

Interface               Admin Link Proto    Local                 Remote

esi                     up    up

---------------------------------------------------------------------------------------------------------------------

FABRIC VALIDATION

jcluser@spine1> show evpn database

Instance: default-switch

VLAN  DomainId  MAC address        Active source                  Timestamp        IP address

5010       00:00:5e:00:01:01  05:00:00:07:d0:00:00:13:92:00  May 17 10:44:38  10.10.10.254

5010       00:50:56:be:0d:ff  00:11:22:33:44:55:66:77:88:99  May 17 10:47:33  10.10.10.9

5010       00:50:56:be:29:5c  00:11:22:33:44:55:66:77:88:99  May 17 10:44:38

5010       00:50:56:be:e3:a7  00:11:22:33:44:55:66:77:88:99  May 17 10:44:38

5010       2c:6b:f5:2c:1c:f0  2.2.2.2                        May 17 10:44:27  10.10.10.2

5010       2c:6b:f5:65:8a:f0  irb.10                         May 17 10:44:38  10.10.10.1

5010       fe:54:00:86:e1:0a  00:11:22:33:44:55:66:77:88:99  May 18 10:28:46

5020       00:00:5e:00:01:01  05:00:00:07:d0:00:00:13:9c:00  May 17 10:44:38  11.11.11.254

5020       00:50:56:be:42:e7  00:11:22:33:44:55:66:77:88:aa  May 17 10:45:33  11.11.11.9

5020       2c:6b:f5:2c:1c:f0  2.2.2.2                        May 17 10:44:27  11.11.11.2

5020       2c:6b:f5:65:8a:f0  irb.20                         May 17 10:44:38  11.11.11.1

jcluser@spine1> show route table bgp.evpn.0

bgp.evpn.0: 49 destinations, 49 routes (49 active, 0 holddown, 0 hidden)

+ = Active Route, - = Last Active, * = Both

1:1.1.1.1:0::112233445566778899::FFFF:FFFF/192 AD/ESI

*[EVPN/170] 1d 02:37:22

Indirect

1:1.1.1.1:0::1122334455667788aa::FFFF:FFFF/192 AD/ESI

*[EVPN/170] 1d 00:50:19

Indirect

1:1.1.1.1:0::05000007d00000139200::FFFF:FFFF/192 AD/ESI

*[EVPN/170] 23:48:29

Indirect

1:1.1.1.1:0::05000007d00000139c00::FFFF:FFFF/192 AD/ESI

*[EVPN/170] 23:48:29

Indirect

1:1.1.1.1:2000::112233445566778899::0/192 AD/EVI

*[EVPN/170] 1d 02:37:23

Indirect

1:1.1.1.1:2000::1122334455667788aa::0/192 AD/EVI

*[EVPN/170] 1d 00:50:21

Indirect

1:2.2.2.2:0::112233445566778899::FFFF:FFFF/192 AD/ESI

*[BGP/170] 1d 02:37:22, localpref 100, from 2.2.2.2

AS path: I, validation-state: unverified

>  to 16.0.0.2 via ge-0/0/0.0

1:2.2.2.2:0::1122334455667788aa::FFFF:FFFF/192 AD/ESI

*[BGP/170] 1d 00:50:20, localpref 100, from 2.2.2.2

AS path: I, validation-state: unverified

>  to 16.0.0.2 via ge-0/0/0.0

1:2.2.2.2:0::05000007d00000139200::FFFF:FFFF/192 AD/ESI

*[BGP/170] 1d 02:33:32, localpref 100, from 2.2.2.2

AS path: I, validation-state: unverified

>  to 16.0.0.2 via ge-0/0/0.0

1:2.2.2.2:0::05000007d00000139c00::FFFF:FFFF/192 AD/ESI

*[BGP/170] 1d 00:44:08, localpref 100, from 2.2.2.2

AS path: I, validation-state: unverified

>  to 16.0.0.2 via ge-0/0/0.0

1:2.2.2.2:2000::112233445566778899::0/192 AD/EVI

*[BGP/170] 1d 02:37:23, localpref 100, from 2.2.2.2

AS path: I, validation-state: unverified

>  to 16.0.0.2 via ge-0/0/0.0

1:2.2.2.2:2000::1122334455667788aa::0/192 AD/EVI

*[BGP/170] 1d 00:50:21, localpref 100, from 2.2.2.2

AS path: I, validation-state: unverified

>  to 16.0.0.2 via ge-0/0/0.0

2:1.1.1.1:2000::5010::00:00:5e:00:01:01/304 MAC/IP

jcluser@spine1> show ethernet-switching table

MAC flags (S - static MAC, D - dynamic MAC, L - locally learned, P - Persistent static, C - Control MAC

SE - statistics enabled, NM - non configured MAC, R - remote PE MAC, O - ovsdb MAC,

B - Blocked MAC)

Ethernet switching table : 8 entries, 8 learned

Routing instance : default-switch

Vlan                MAC                 MAC       GBP    Logical                SVLBNH/      Active

name                address             flags     tag    interface              VENH Index   source

anurag              00:00:5e:00:01:01   DRP              esi.604                             05:00:00:07:d0:00:00:13:92:00

anurag              00:50:56:be:0d:ff   DLR              ae0.0

anurag              00:50:56:be:29:5c   DLR              ae0.0

anurag              00:50:56:be:e3:a7   DLR              ae0.0

anurag              2c:6b:f5:2c:1c:f0   DR               vtep.32769                          2.2.2.2

shekhar             00:00:5e:00:01:01   DRP              esi.614                             05:00:00:07:d0:00:00:13:9c:00

shekhar             00:50:56:be:42:e7   DLR              ae1.0

shekhar             2c:6b:f5:2c:1c:f0   DR               vtep.32769                          2.2.2.2

jcluser@spine1> show route

inet.0: 14 destinations, 15 routes (14 active, 0 holddown, 0 hidden)

Limit/Threshold: 1048576/1048576 destinations

+ = Active Route, - = Last Active, * = Both

1.1.1.1/32         *[Direct/0] 1d 04:30:08

>  via lo0.0

2.2.2.2/32         *[BGP/170] 1d 03:39:33, localpref 100

AS path: 6000 I, validation-state: unverified

>  to 16.0.0.2 via ge-0/0/0.0

[BGP/170] 1d 03:39:33, localpref 100

AS path: 6000 I, validation-state: unverified

>  to 17.0.0.2 via ge-0/0/1.0

10.10.10.0/24      *[Direct/0] 23:51:28

>  via irb.10

10.10.10.1/32      *[Local/0] 23:51:28

Local via irb.10

10.10.10.9/32      *[EVPN/7] 23:48:32

>  via irb.10

10.10.10.254/32    *[Local/0] 23:51:28

Local via irb.10

11.11.11.0/24      *[Direct/0] 23:51:28

>  via irb.20

11.11.11.1/32      *[Local/0] 23:51:28

Local via irb.20

11.11.11.9/32      *[EVPN/7] 23:51:21

>  via irb.20

11.11.11.254/32    *[Local/0] 23:51:28

Local via irb.20

16.0.0.0/30        *[Direct/0] 1d 04:30:08

>  via ge-0/0/0.0

16.0.0.1/32        *[Local/0] 1d 04:30:08

Local via ge-0/0/0.0

17.0.0.0/30        *[Direct/0] 1d 04:30:08

>  via ge-0/0/1.0

17.0.0.1/32        *[Local/0] 1d 04:30:08

Local via ge-0/0/1.0